EXPERIMENT 1

AIM: Write a python program to Simulate continuous and discrete sine wave signal

Code:

import numpy as np

import matplotlib.pyplot as plt

def simulate_continuous_sine_wave(time, amplitude, frequency, phase):

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

def simulate_discrete_sine_wave(num_samples, sampling_frequency, amplitude, frequency, phase):

time = np.arange(num_samples) / sampling_frequency

sine_wave = amplitude * np.sin(2 * np.pi * frequency * time + phase)

return sine_wave

# Define the time range for the continuous sine wave signal

time = np.linspace(0, 1, 1000)  # Time range from 0 to 1 second

# Define the number of samples, sampling frequency, and parameters for the discrete sine wave signal

num_samples = 100          # Number of samples

sampling_frequency = 10    # Sampling frequency in Hz

amplitude = 1              # Amplitude of the sine wave

frequency = 2              # Frequency of the sine wave in Hz

phase = 0                  # Phase angle of the sine wave in radians

# Simulate the continuous sine wave signal

continuous_sine_wave = simulate_continuous_sine_wave(time, amplitude, frequency, phase)

# Simulate the discrete sine wave signal

discrete_sine_wave = simulate_discrete_sine_wave(

num_samples,

sampling_frequency,

amplitude,

frequency,

phase

)

# Plot and display the continuous and discrete sine wave signals

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(time, continuous_sine_wave)

plt.title('Continuous Sine Wave Signal')

plt.xlabel('Time (s)')

plt.ylabel('Amplitude')

plt.subplot(2, 1, 2)

plt.stem(discrete_sine_wave)

plt.title('Discrete Sine Wave Signal')

plt.xlabel('Sample')

plt.ylabel('Amplitude')

plt.tight_layout()

plt.show()

# Save the sine wave signal arrays (optional)

# np.savetxt('continuous_sine_wave.txt', continuous_sine_wave, delimiter=',')

# np.savetxt('discrete_sine_wave.txt', discrete_sine_wave, delimiter=',')

OUTPUT: